GRLIB -- Hires C64 graphics library

Last updated: 3/14/01 sjudd@ffd2.com

grlib is a simple 2D graphics library for you to use in your programs.  It's

really just the old BLARG routines, fixed up a little and with a jump table

added.  It's really easy to use, and there's a little test program included

to show you how all the routines work.  The file "jumptab.s" contains the

grlib jump table and all the constants it uses, so include it in any assembly

program and off you go.

To use it, just load to $C000.  That's it.

Here is a brief description of the routines and how to call them.  For more

information check out the BLARG docs (and the article in disC=overy issue 2).

$C000	InitGr		Initialize the graphics routines: set origin to

upper-left corner, bitmap at $E000, color map

to $FC00, and MODE 17 graphics (all explained later!)

Arguments:	none

$C003	SetOrg		Set the screen origin (0,0).  Normally this origin

is the upper-left corner of the screen.

Arguments:	.X/.Y contain the new X/Y origin coordinates

$C006	GRON		Turn GRaphics ON

Arguments:	If .A = 0 then turn bitmap on

Otherwise, initialize colormap to .A, clear bitmap,

and turn bitmap on.

$C009	GROFF		Turn graphics off -- restores graphics state to

whatever it was when GRON was called.

Arguments:	None

$C00C	SETCOLOR	Set drawing color to foreground or background

Arguments:	.A = 0 -> Background color

.A = 1 -> Foreground color

$C00F	MODE		Change drawing mode or set bit mask.  Three modes

are available:

Arguments	.A = 16 -> Mode 16, optimized for the SCPU (bitmap

in VIC bank 2 and optimized mode set)

.A = 17 -> Mode 17, normal mode (VIC bank 3, text

at $0400)

.A = 18 -> Mode 18, double-buffer mode (VIC banks

1 and 2, swap buffers with SWAPBUF)

Otherwise, set BITMASK to .A -- BITMASK is logical

AND-ed with the bitmap pattern when drawing lines.

$C012	SETBUF		Set the current drawing buffer, in double buffer mode

Arguments:	.X = 0 -> Swap buffers

.X = 1 -> Buffer 1 ($E000)

.X = 2 -> Buffer 2 ($A000)

$C015	SWAPBUF		Swap the *DISPLAYED* buffer.  The idea is to draw

into the back buffer, then swap it forward when it's

ready to be displayed.

Arguments:	none

PLOT, LINE, and CIRCLE all use *signed* *16-bit* coordinates, stored in

zero page before calling the routine.  Don't forget and assume that the upper

eight bits will be zero!

$C018	PLOT		Plot the point in X1, Y1 (zero page locations), offset

by ORGX/ORGY.

PLOT is much slower than it needs to be (should just

use a lookup table).

Arguments:	X1 = $02/$03: 16-bit X-coordinate

Y1 = $04/$05: 16-bit Y-coordinate

$C01B	PLOTABS		Like PLOT, but ignores ORGX/ORGY

Arguments:	Same as PLOT

$C01E	LINE		Draw a line from (X1,Y1) to (X2,Y2).  Endpoints can

be off the visible screen.  Line pattern is determined

by BITMASK, set using MODE command (see above).

Arguments:	(X1,Y1) = ($02,$04)

(X2,Y2) = ($06,$08) 16-bit line endpoints

$C021	CIRCLE		Draw a circle of radius RADIUS, centered at X1,Y1.

The circle routine is very fast, and doesn't mind

points off the visible screen.

Arguments:	(X1,Y1) = ($02,$04): 16-bit center coordinates

RADIUS = $10: Radius of circle (8-bits)

GRLIB Jump table and memory usage:

*

* grlib jump table

*

InitGr   = $C000

SetOrg   = Init+3

GRON     = SetOrg+3

GROFF    = GRON+3

SETCOLOR = GROFF+3

MODE     = SETCOLOR+3

SETBUF   = MODE+3

SWAPBUF  = SETBUF+3

PLOT     = SWAPBUF+3

PLOTABS  = PLOT+3

LINE     = PLOTABS+3

CIRCLE   = LINE+3

*

* grlib constants

*

X1       = $02

Y1       = $04

X2       = $06

Y2       = $08

DX       = $0A

DY       = $0C

ROW      = $0D            ;Bitmap row

COL      = $0E            ;and column

INRANGE  = $0F            ;Range check flag

RADIUS   = $10

POINT    = $FD            ;Bitmap pointer